University of Asia Pacific

Department of Computer Science & Engineering

Course Title   :   Microprocessors and Microcontrollers Lab

Course Code :  CSE 316

Report No        :  03

Report Title     :  Touch Sensor Door Lock System

Date                     : 26 - 04 - 2026

c

Submitted By :                    Submitted To :

Name : Md. Osman Goni            Khadiza Sultana Sayma

Reg. No : 23101012                       Lecturer, Department of CSE

Section :  A1                                      University Of Asia Pacific.

- Experiment Name

Touch Sensor Door Lock System

## 2. Objective

To design and implement a touch/push-button based door lock system using Arduino, where a servo motor locks/unlocks the door and a buzzer provides different sound indications for each action.

## 3. Apparatus / Hardware & Software Requirements

- Hardware

- Arduino Uno

- Push button / Touch sensor ( digital input )

- Servo motor

- Buzzer

- Breadboard

- Jumper wires

- USB cable / 5V supply

Software

- Arduino IDE

- Arduino Servo library (#include <Servo.h>)

4. Circuit Diagram / Schematic

5. Code / Assembly Program

#include <Servo.h>

Servo myServo;

const int PushPin = 2;

const int buzzerPin = 8;

const int servoPin = 9;

bool doorLocked = true;

void setup() {

pinMode(PushPin, INPUT);

pinMode(buzzerPin, OUTPUT);

myServo.attach(servoPin);

myServo.write(0); // Initially locked position

}

void loop() {

int touchState = digitalRead(PushPin);

if (touchState == HIGH) {

if (doorLocked) {

// Unlock door

myServo.write(90);

tone(buzzerPin, 200, 500000);

delay(200);

noTone(buzzerPin);

doorLocked = false;

} else {

// Lock door

myServo.write(0);

tone(buzzerPin, 500, 400000);

delay(400);

noTone(buzzerPin);

doorLocked = true;

}

delay(1000);

}

}

## 6. Output / Observations

- The system starts in locked position (servo at 0°).

- When the touch/push input is detected HIGH, the Arduino toggles the door state:

  - If locked → it unlocks and gives a buzzer tone.

  - If unlocked → it locks and gives a different buzzer tone.

- The system waits briefly (delay(1000)) to avoid rapid repeated toggling.

## 7. Result

After implementing and testing the system, the following observations were recorded:

Unlock Action (when doorLocked = true):

When the button was pressed while the door was locked, the servo rotated to 90° (unlock position). The buzzer produced a low-frequency tone (200 Hz), indicating successful unlocking. The door state changed to unlocked and the system responded correctly.

Lock Action (when doorLocked = false):

When the button was pressed again while the door was unlocked, the servo rotated back to 0° (lock position). The buzzer produced a higher-frequency tone (500 Hz), indicating successful locking. The door state changed back to locked and the cycle repeated properly.

## 8. Conclusion

This project provided practical experience in building a simple access-control system using Arduino. It strengthened understanding of digital input reading, controlling a servo motor for mechanical movement, and using a buzzer for audio feedback. The experiment also improved skills in implementing toggle logic using a state variable (doorLocked) and coordinating multiple outputs reliably.

Overall, the touch sensor door lock system worked as expected: each press alternated between lock and unlock states with clear visual/mechanical action from the servo and distinct sound feedback from the buzzer. This setup offers a strong foundation for future enhancements such as adding RFID, keypad security, password control, or wireless access features.