University of Asia Pacific

Department of Computer Science & Engineering

Course Title   :   Microprocessors and Microcontrollers Lab

Course Code :  CSE 316

Report No        :  06

Report Title     :  Remote AC Bulb Control

Date                     : 25 - 05 - 2026

c

Submitted By :                    Submitted To :

Name : Md. Osman Goni            Khadiza Sultana Sayma

Reg. No : 23101012                       Lecturer, Department of CSE

Section :  A1                                      University Of Asia Pacific.

- Experiment Name

Remote AC Bulb Control

## Objective

To control an AC bulb wirelessly using an IR remote, Arduino, a relay module, and an IR receiver, so the bulb can be switched ON or OFF at a distance.

## Apparatus / Hardware & Software Requirements

Hardware:

- Arduino UNO board

- IR Receiver sensor

- IR Remote (as shown, standard NEC protocol remote)

- Relay Module (suitable for AC voltage)

- AC Bulb with holder and wiring

- 9V Battery for relay/bulb

- Breadboard and connecting wires

- USB Cable for Arduino

Software:

- Arduino IDE

- IRremote library for Arduino (#include <IRremote.hpp>)

4. Circuit Diagram / Schematic

- Code / Assembly Program

#include <IRremote.hpp>

const int relayPin = 3;

const int rcvPin = 4;

void setup() {

pinMode(relayPin, OUTPUT);

digitalWrite(relayPin, LOW);

Serial.begin(9600);

IrReceiver.begin(rcvPin, ENABLE_LED_FEEDBACK);

Serial.println("IR receiver ready...");

}

void loop() {

if (IrReceiver.decode()) {

unsigned long value = IrReceiver.decodedIRData.decodedRawData;

Serial.print("IR code: ");

Serial.println(value);

switch (value) {

// ON button (replace with your remote code)

case 4010852096:

Serial.println("Bulb ON");

digitalWrite(relayPin, HIGH);

break;

// OFF button

case 4077698816:

Serial.println("Bulb OFF");

digitalWrite(relayPin, LOW);

break;

default:

Serial.println("Unknown button");

break;

}

IrReceiver.resume();

}

delay(200);

}

- Output / Observations

- On startup, Serial Monitor displays:
IR receiver ready...

- When a remote button is pressed, the Arduino prints the received value:
IR code: <number>

- If the received code matches a stored ON/OFF code, it prints:

  - Bulb ON and sets relay pin HIGH

  - Bulb OFF and sets relay pin LOW

- If a different button is pressed, it prints:
Unknown button

## 7. Result

## After implementing and testing the system, the following observations were recorded:

## Pressing the programmed ON button (code 4010852096) energized the relay and the AC bulb turned ON.

## Pressing the programmed OFF button (code 4077698816) de-energized the relay and the AC bulb turned OFF.

## Other buttons produced “Unknown button” until their codes were added to the switch cases.

## The system successfully demonstrated reliable remote switching, confirmed by Serial Monitor outputs and the bulb’s response.

## 8. Conclusion

This project successfully implemented remote AC load control using an IR remote and Arduino. It demonstrated IR signal decoding with the IRremote library and safe switching of higher-voltage loads using a relay. The Serial Monitor provided a clear method to capture remote button codes and map them to ON/OFF actions.

With minor enhancements (adding more button functions, debouncing/repeat-code handling, and using a proper relay module enclosure), the design can be expanded into a practical home automation-style controller.